How to install the Programming Cable Driver (WIN 10 System)

This install guideline are compatible for the programming cable of BAOFENG ALL Analog Radio、TYT ALL Analog Radio, Radioddity ALL Analog Radio, and Radioddity GD-55 PLUS、TYT MD-680D/680.

- Get into “Window Setting”, then search “Device Manager”

- Open “Device Manager”, and unfold Port (COM & LPT) to check the existing port.

- Connect the radio and the computer via cable, then have a check if there is new devices installed. If nothing happen, try to unplug and plug in the computer for several times.

- In this step, there are two situation:

- If the new device do not come with the exclamation symbol, it means the driver installed successfully. Then you can start to download the programming software to program you radio.

- If so, that means the driver goes wrong. In this case, please follow the step below to fix the issue.

- Firstly, right click the error device and click “Uninstall device”.

- After uninstall the device, then unplug the cable. Then, download and install the Cable Driver through this link: https://radioddity.myshopify.com/pages/others-download

- And again, connect the radio and the computer via cable. Then check out the “Port” in “Device Manager”, if there is no exclamation symbol that means the cable driver runs well.

- Finally, you can go forward to the next step——Download the programming software to program your radio.

- If the driver still goes wrong, please feel free to contact us for assistance.(radioddity.com)